CNS 3370

Program 5 – Data Reduction with the STL

Date Due: See Syllabus

In this program you will analyze digitized data for pulses. The data comes from an actual digitizer that reports voltages (except all the data has been reversed by the hardware – you must multiply all values by -1 before proceeding). Here is a graph of one of the sample data files (which consist of integers separated by whitespace) after negating the values:

The data is quite jagged, so we prefer a smooth version instead. Starting with the 4th point of the file, and ending with the 4th from the last, replace each of those points with the following average (the current point in question is pointed to by “iter”):

(iter[-3] + 2*iter[-2] + 3*iter[-1] + 3*iter[0] + 3*iter[1] + 2*iter[2] + iter[3]) / 15

Here’s what the smoothed data looks like:

The smooth data will be better for detecting pulses (you can see two noticeable ones in the pictures above).

You detect a pulse by looking for a rise over three consecutive points. If the rise (xi+2 – xi) exceeds vt, (for “voltage threshold” – supplied by an input parameter), then a pulse begins at position i. Move forward through the data until it starts to decrease before looking for another pulse.

Write a program that process all files with a “.dat” extension in the current directory. There can be an arbitrary number of such files, and an arbitrary number of sample data values. Print out where pulses are found, and also the “area” underneath the pulses. The area is merely the sum of the values starting at the pulse start and going for width samples (another input parameter), or until the start of the next pulse, whichever comes first. Use the original, unsmooth data to compute the area, however.

There is one “gotcha”. Sometimes it is hard to distinguish “piggyback” pulses (where a pulse is adjacent to another) from a wide pulse with some variation to it. To distinguish these, we use the following parameters:

| Parameter | Description |
| --- | --- |
| drop_ratio | A number less than 1 |
| below_drop_ratio | The number of values less than drop_ratio |
| pulse_delta | The gap between pulses to look for piggybacks |

When pulses begin within pulse_delta positions of each other, find how many points between the peak of the first pulse and the start of the second pulse fall below drop_ratio times the peak of the first pulse. If the number exceeds below_drop_ratio, omit the first pulse from further consideration (it is not a pulse of interest).

Read all parameters from an .ini file like the following at program startup:

vt=100

width=100

pulse_delta=15

drop_ratio=0.75

below_drop_ratio=4

All parameters are necessary and none others are allowed. Enforce this.

If a .dat file has no pulses, ignore it. For those that do, report them and their areas as follows:

2_Record2308.dat: 1019 1030

3540 22916

2_Record3388.dat: 1035

41264

as_ch01-0537xx_Record1042.dat: 1050

8228

The first .dat file has pulses of interest at (zero-based) positions 1019 and 1030, with respective areas 3540 and 22916. The .dat files are in this ZIP file.

Implementation Issues

Use loops only when necessary. Use STL algorithms such as copy, transform, and accumulate, along with istream_iterators to make short work of this program. I have 8 loops and 8 calls to STL algorithms. How are you going to discover all the .dat files in the current directory? Verify that the .ini file is present and valid. FYI, my program is 228 lines.